Kia Rio 2013 D 1.1 – Fuses and Relays

Component Location

1. PCB fuse & relay box (Head lamp low/ hi, Fuel pump #1, Fuel heater, HAC, Engine control, ESS, Front wiper, Rain sensor, Blower, Cooling fan low/ hi, Horn, Buglar alarm horn, Start relay)

Components

Inspection

Replacement of EMS box

| 1. | Disconnect the negative (-) battery terminal. |
| --- | --- |

| 2. | Push hooks in the engine room relay box out to the arrow direction and put up the EMS box assembly (A). |
| --- | --- |

| 3. | Disconnect the connector (A) and remove the EMS box assembly (B). |
| --- | --- |

Fuse Inspection

| 1. | Be sure there is no play in the fuse holders, and that the fuses are held securely. |
| --- | --- |

| 2. | Are the fuse capacities for each circuit correct? |
| --- | --- |

| 3. | Are there any blown fuses? If a fuse is to be replaced, be sure to use a new fuse of the same capacity. Always determine why the fuse blew first and completely eliminate the problem before installing a new fuse. |
| --- | --- |

Multi Fuse

Multi Fuse is for optimizing the engine room package.

-; Multi fuse (A) is needed to replace entirely when there is damage to only one fuse.. -; When replace the multi fuse, refer to the "Engine compartment - component location" diagram exactly.. -; Use the multi fuse capacities for each circuit correctly..

Components

Fuse Inspection

| 1. | Be sure there is no play in the fuse holders, and that the fuses are held securely. |
| --- | --- |

| 2. | Are the fuse capacities for each circuit correct? |
| --- | --- |

| 3. | Are there any blown fuses? If a fuse is to be replaced, be sure to use a new fuse of the same capacity. Always determine why the fuse blew first and completely eliminate the problem before installing a new fuse. |
| --- | --- |

Removal

Passenger Compartment Junction Box

| 1. | Disconnect the negative(-) battery terminal. |
| --- | --- |

| 2. | Remove the crash pad lower panel. (Refer to the BD group - "Crash pad") |
| --- | --- |

| 3. | Disconnect the connectors from the fuse side of the interior junction box. |
| --- | --- |

| 4. | Remove the interior junction box(A) after loosening the mounting nuts (2EA). |
| --- | --- |

| 5. | Disconnect the connectors from the back side of the smart junction box. |
| --- | --- |

Installation

| 1. | Install the interior junction box. |
| --- | --- |

| 2. | Install the crash pad lower panel. |
| --- | --- |

| 3. | Check that all system operates normally. |
| --- | --- |